Balikburnu: 37.9903°N, 38.1990°E , length 5 m, obliquity30°  3 March-9 September

Figure  1. The Balikburnu creepmeter was installed across a 5.3 m rupture offset in sediments at the side of a road (the stalks are from a tobacco plants) south of the village of Balikburnu. A local recording system records the data.  It is powered by a bank of D-cells.

Figure 2 View north showing 5.3 m sinistal offset of track at Balikburnu

Data from this site (Figure 3) show minor dextral contraction symptomatic of gravitational collapse of the open fissures present at the time of installation.  In iearly September, because of  the absence of afterslip, and because the aqueduct that had been destroyed by the earthquake had now been repaired, the instrument was removed.

Figure 3  The Balikburnu extensometer recorded no afterslip.  We attribute the apparent 2 mm dextral signal to closure of the gaping fissures at this location.  A nearby drainage ditch had been repaired and was active during a September visit and it was decided to cease operationas at this location.